# EV Battery Management System (BMS) with SOC Estimation Using MATLAB and Simulink

Project guide: design, code, Simulink model, tests, analysis, troubleshooting (The project report and the viva Q&A are in separate files.)

## 0. Read this first: what has and has not been tested

| Item | Status |
| --- | --- |
| MATLAB .m files | Written but not run in MATLAB. No MATLAB or Octave was available when this was produced. |
| Algorithm and scenario design | Cross-checked by an independent Python re-implementation of the same equations. The numbers in sections 8–9 come from that cross-check. Your MATLAB output should match them to rounding. |
| create_EV_BMS_model.m (Simulink generator) | Not run in Simulink. Block list, parameters and all 118 connections were checked by a script that parses the file: no dangling inputs, no input driven twice, no algebraic loop, every workspace variable defined. This does not prove that your MATLAB release accepts every parameter name. Section 12 lists likely fixes. |
| EKF | Prototyped in Python on a model-matched synthetic cell. This shows the algorithm converges in a best case. It is not validated and says nothing about accuracy on a real battery. |
| Battery data | The OCV curve, R0, thermal values and every threshold are illustrative, not measured and not from a datasheet. |

First thing to do: run EV_BMS_Main and confirm the table at the end says OVERALL: PASS. Then run create_EV_BMS_model and verify_simulink_vs_matlab.

## 1. Project overview and architecture

Goal. Simulate a 48 V, 50 Ah EV battery pack, estimate its State of Charge (SOC) by Coulomb counting, watch voltage, current and temperature, and raise protection flags for overvoltage (OV), undervoltage (UV), overcurrent (OC) and overtemperature (OT).

Two stages.

- Stage A: synthetic V, I, T signals, with each fault injected in its own time window. This tests the fault logic in isolation.

- Stage B: a simplified battery model (V = OCV(SOC) − I·R0 plus an I²R thermal model) driven by a load-current profile. The same logic is tested under realistic coupling: charging raises voltage, discharge lowers it, current heats the pack.

Pack. 13 cells in series (13S) of generic NMC cells. 13 × 3.7 V ≈ 48 V nominal. Limits: 3.0 V to 4.2 V per cell, so 39.0 V to 54.6 V.

+--------------------------------------------------------+
  Load current   |  Protection_Control  (simulated contactor + latch)     |
  profile ------>|  I_actual = I_cmd x (contactor closed)                 |
                 +-------------------------+------------------------------+
                                           | I_actual
                                           v
  T_ambient ---------------------> +-------------+  V, I, T, SOC_true
                                   | Battery_    |-------------------------+
                                   | Model       |                         |
                                   +-------------+                         v
  Stage A synthetic V,I,T  ------------------------------------>  [Stage switch]
                                                                           |
                                                                           v
                                                                +------------------+
                                                                | Battery_Monitoring|-- I_meas --> SOC_Estimator
                                                                | (+ sensor offset) |                (Coulomb counting)
                                                                +--------+---------+
                                                                         | V, I, T
                                                                         v
                                                                +------------------+     Fault_any
                                                                | Fault_Detection  |------------------+
                                                                | OV UV OC OT ANY  |                  |
                                                                +--------+---------+                  |
                                                                         |                            |
                                                                         v                            |
                                                                +------------------+                  |
                                                                |  Visualization   |                  |
                                                                |  scopes/displays |                  |
                                                                +------------------+                  |
   (fault latch inside Protection_Control closes the loop) <-----------------------------------------+

Files

| File | Role |
| --- | --- |
| bms_params.m | All parameters, thresholds, OCV table. Edit here. |
| coulomb_counting.m | SOC estimator function |
| battery_voltage_model.m | V = OCV(SOC) − I·R0 |
| bms_fault_detection.m | OV / UV / OC / OT / combined flags |
| bms_scenarios.m | The 11 test scenarios |
| simulate_bms.m | Time loop: plant, estimator, faults, contactor |
| bms_stageA_signals.m | Stage A synthetic signals |
| plot_bms_results.m | Six-panel plots |
| EV_BMS_Main.m | Main script: runs everything, prints PASS/FAIL, plots |
| ekf_soc_estimator.m, EV_BMS_EKF_demo.m | Optional advanced EKF module |
| bms_simulink_init.m | Defines workspace variables for the Simulink model |
| create_EV_BMS_model.m | Builds and saves EV_BMS_SOC.slx |
| verify_simulink_vs_matlab.m | Compares Simulink with MATLAB for all scenarios |

## 2. Mathematical equations and assumptions

Sign convention: current I > 0 is discharge, I < 0 is charge.

Coulomb counting

SOC(k) = SOC(k-1) - ( I(k) * dt / (3600 * Q) ) * 100        [%]

I in A, dt in s, Q in Ah. I·dt/3600 converts A·s to Ah. Dividing by Q gives a fraction, and ×100 gives percent. SOC is clipped to 0–100 %.

Terminal voltage

V = OCV(SOC) - I * R0

Discharge (I > 0) lowers the terminal voltage, charge (I < 0) raises it.

Thermal model (lumped)

Cth * dT/dt = I^2 * R0 - (T - T_amb) / Rth
T(k) = T(k-1) + dt/Cth * ( I^2*R0 - (T(k-1) - T_amb)/Rth )

Steady state: T = T_amb + I²·R0·Rth.

Fault logic

OV  = V > V_max          UV = V < V_min
OC  = |I| > I_max        OT = T > T_max
ANY = OV or UV or OC or OT

Simulink gain (continuous Integrator, I in A, time in s, SOC in %):

K = -100 / (3600 * Q)  = -100 / (3600 * 50) = -5.5556e-4  % per (A*s)
SOC(t) = SOC0 + integral( K * I dt )

Assumptions and limits

- OCV depends on SOC only (no temperature, ageing or hysteresis).

- R0 is constant. In reality it rises at low SOC, low temperature and with ageing.

- No RC branch, so voltage reacts instantly to current steps.

- The thermal model is lumped, with a single temperature and a single Rth/Cth. Cth is deliberately small so heating shows up within minutes.

- Capacity Q is constant (no State of Health).

- Protection thresholds are illustrative. Real limits come from the cell datasheet.

- Measurement noise, debounce timers and hysteresis are left out for clarity.

## 3. MATLAB code

Put every file in one folder. Base MATLAB is enough, with no toolboxes.

### 3.1 bms_params.m

function P = bms_params()
%BMS_PARAMS  Central configuration for the EV BMS / SOC project.
%
%   P = bms_params() returns a struct with every configurable parameter.
%   All other files read their settings from here, so change values HERE.
%
%   SIGN CONVENTION : current > 0 = DISCHARGE, current < 0 = CHARGE.
%   UNITS           : V, A, Ah, ohm, degC, s, J/K, K/W.  SOC is in PERCENT.
%
%   IMPORTANT: every number below is ILLUSTRATIVE. Real protection limits,
%   R0, thermal values and the OCV curve must come from the cell
%   manufacturer's datasheet and from lab tests of your actual cells.

    % ---------------- Pack definition (illustrative 13S NMC pack) -----------
    P.Ns        = 13;      % cells in series  (13 x 3.7 V = 48.1 V nominal)
    P.V_nom     = 48;      % V   nominal pack voltage (documentation only)
    P.Q_Ah      = 50;      % Ah  pack capacity

    % ---------------- Initial conditions ------------------------------------
    P.SOC_init     = 80;   % %    true initial SOC of the battery
    P.SOC_est_init = [];   % %    estimator start value; [] = same as SOC_init
                           %      (set e.g. 60 to see the effect of a wrong start)
    P.T_init       = 25;   % degC initial pack temperature
    P.T_amb        = 25;   % degC ambient temperature

    % ---------------- Protection thresholds (illustrative) ------------------
    P.V_max = 54.6;        % V   = 13 x 4.20 V   overvoltage limit
    P.V_min = 39.0;        % V   = 13 x 3.00 V   undervoltage limit
    P.I_max = 100;         % A   magnitude limit, applies to charge AND discharge
    P.T_max = 60;          % degC overtemperature limit

    % ---------------- Equivalent-circuit model (Stage B) --------------------
    P.R0  = 0.025;         % ohm  pack internal resistance (constant, simplified)
    P.Rth = 0.15;          % K/W  pack-to-ambient thermal resistance
    P.Cth = 8000;          % J/K  pack heat capacity (illustrative, deliberately
                           %      small so thermal effects show up in minutes)

    % ---------------- Sensor / simulation settings --------------------------
    P.I_offset = 0;        % A   current-sensor offset seen ONLY by the BMS
    P.dt       = 1;        % s   sampling time
    P.t_end    = 3600;     % s   default duration (scenarios override this)
    P.protection_enable = true; % simulated contactor logic (scenario may override)

    % ---------------- Illustrative OCV-SOC curve ----------------------------
    % Per-cell open-circuit voltage of a generic NMC cell (NOT measured data).
    % Replace with your own table: rest the cell 1-2 h at each SOC step,
    % record the settled voltage, and paste the numbers here.
    P.SOC_bp    = [0   5    10   20   30   40   50   60   70   80   90   100];
    P.OCV_cell  = [3.00 3.30 3.45 3.57 3.64 3.70 3.77 3.85 3.94 4.04 4.13 4.20];
    P.OCV_pack  = P.Ns * P.OCV_cell;   % pack-level curve used by the model
end

### 3.2 coulomb_counting.m

function SOC = coulomb_counting(I, dt, Q_Ah, SOC0)
%COULOMB_COUNTING  SOC estimation by current integration.
%
%   SOC(k) = SOC(k-1) - ( I(k) * dt / (3600 * Q_Ah) ) * 100
%
%   Inputs
%     I     current in A, positive = discharge, negative = charge
%           - vector (N samples): returns the whole SOC history
%           - scalar            : single-step update, SOC0 is the PREVIOUS SOC
%     dt    sampling time in s            (scalar > 0)
%     Q_Ah  battery capacity in Ah        (scalar > 0)
%     SOC0  initial SOC in %, 0..100      (scalar)
%
%   Output
%     SOC   in %, saturated to [0, 100].
%           Vector mode: SOC(1) = SOC0 (the state at t = 0), then
%           SOC(k) is computed with I(k) for k = 2..N.
%
%   Unit check: A * s / (3600 s/h) = Ah.  Ah / Q_Ah = fraction.  x100 = %.

    % ---------------- input validation --------------------------------------
    if ~isnumeric(I) || ~isreal(I) || isempty(I) || any(~isfinite(I(:)))
        error('coulomb_counting:badCurrent', ...
              'I must be a non-empty, real, finite numeric array (A).');
    end
    if ~isscalar(dt) || ~isnumeric(dt) || ~isfinite(dt) || dt <= 0
        error('coulomb_counting:badDt', 'dt must be a positive scalar (s).');
    end
    if ~isscalar(Q_Ah) || ~isnumeric(Q_Ah) || ~isfinite(Q_Ah) || Q_Ah <= 0
        error('coulomb_counting:badCapacity', ...
              'Q_Ah must be a positive scalar (Ah).');
    end
    if ~isscalar(SOC0) || ~isnumeric(SOC0) || ~isfinite(SOC0) ...
            || SOC0 < 0 || SOC0 > 100
        error('coulomb_counting:badSOC0', ...
              'SOC0 must be a scalar between 0 and 100 (%%).');
    end

    k_gain = dt / (3600 * Q_Ah) * 100;      % % of SOC per ampere of current

    % ---------------- single-step mode --------------------------------------
    if isscalar(I)
        SOC = min(max(SOC0 - I * k_gain, 0), 100);
        return;
    end

    % ---------------- vector mode -------------------------------------------
    I   = I(:);
    N   = numel(I);
    SOC = zeros(N, 1);
    SOC(1) = SOC0;
    for k = 2:N
        SOC(k) = SOC(k-1) - I(k) * k_gain;
        SOC(k) = min(max(SOC(k), 0), 100);   % saturation 0..100 %
    end
end

### 3.3 battery_voltage_model.m

function [V_term, OCV] = battery_voltage_model(SOC, I, R0, SOC_bp, OCV_pack)
%BATTERY_VOLTAGE_MODEL  Simplified (R0-only) equivalent-circuit battery.
%
%   V_term = OCV(SOC) - I * R0
%
%   Inputs
%     SOC      state of charge in % (scalar or array, clamped to 0..100)
%     I        current in A, positive = discharge (same size as SOC or scalar)
%     R0       internal resistance in ohm (scalar)
%     SOC_bp   (optional) SOC breakpoints in %,      1xM, increasing
%     OCV_pack (optional) pack OCV at those points in V, 1xM
%              If SOC_bp / OCV_pack are omitted, bms_params() defaults are used.
%
%   Outputs
%     V_term   terminal voltage in V
%     OCV      open-circuit voltage in V
%
%   ASSUMPTIONS
%     1. OCV depends on SOC only (no temperature, no ageing, no hysteresis).
%     2. R0 is constant (in reality it rises at low SOC, low temperature and
%        with ageing).
%     3. No RC (diffusion / polarisation) branch, so the voltage responds
%        instantly to a current step. Real cells relax over seconds-minutes.
%     4. The OCV table is ILLUSTRATIVE, not measured.
%
%   TO USE REAL DATA: replace P.SOC_bp and P.OCV_cell in bms_params.m with a
%   table measured from your cell (rest-and-record at each SOC step), and
%   estimate R0 from the instantaneous voltage step when a current pulse is
%   applied:  R0 = delta_V / delta_I.

    if nargin < 5
        P        = bms_params();
        SOC_bp   = P.SOC_bp;
        OCV_pack = P.OCV_pack;
    end
    if ~isscalar(R0) || ~isnumeric(R0) || R0 < 0
        error('battery_voltage_model:badR0', 'R0 must be a scalar >= 0 (ohm).');
    end
    if numel(SOC_bp) ~= numel(OCV_pack) || any(diff(SOC_bp) <= 0)
        error('battery_voltage_model:badTable', ...
              'SOC_bp must be increasing and the same length as OCV_pack.');
    end

    SOC_clamped = min(max(SOC, SOC_bp(1)), SOC_bp(end));   % no extrapolation
    OCV    = interp1(SOC_bp, OCV_pack, SOC_clamped, 'linear');
    V_term = OCV - I .* R0;
end

### 3.4 bms_fault_detection.m

function [OV, UV, OC, OT, any_fault] = bms_fault_detection(V, I, T, V_max, V_min, I_max, T_max)
%BMS_FAULT_DETECTION  Threshold-based protection flags (logical outputs).
%
%   OV        = V       > V_max      overvoltage
%   UV        = V       < V_min      undervoltage
%   OC        = abs(I)  > I_max      overcurrent (discharge AND charge)
%   OT        = T       > T_max      overtemperature
%   any_fault = OV | UV | OC | OT    combined fault
%
%   Inputs may be scalars or arrays of the same size (V in volts, I in
%   amperes, T in degC). Thresholds are scalars.
%
%   NOTE: the thresholds are illustrative. Real limits come from the cell
%   datasheet. Production BMS code also uses debounce timers and hysteresis
%   so one noisy sample does not trip a fault; that is left out here for
%   clarity (see "Future scope").

    args = {V, I, T};
    for n = 1:3
        if ~isnumeric(args{n}) || any(~isreal(args{n}(:)))
            error('bms_fault_detection:badInput', ...
                  'V, I and T must be real numeric arrays.');
        end
    end
    thr = [V_max, V_min, I_max, T_max];
    if ~isnumeric(thr) || numel(thr) ~= 4 || any(~isfinite(thr))
        error('bms_fault_detection:badThreshold', ...
              'Thresholds must be finite numeric scalars.');
    end
    if V_min >= V_max
        error('bms_fault_detection:badThreshold', 'V_min must be below V_max.');
    end

    OV        = V      > V_max;
    UV        = V      < V_min;
    OC        = abs(I) > I_max;
    OT        = T      > T_max;
    any_fault = OV | UV | OC | OT;
end

### 3.5 bms_scenarios.m

function sc = bms_scenarios(id, P)
%BMS_SCENARIOS  Test-scenario definitions (1..11) for the EV BMS project.
%
%   sc = bms_scenarios(id, P) returns a struct with
%     name, t, I_cmd (A, + = discharge), T_amb (degC), SOC_init (%),
%     T_init (degC), protection (logical), expect (struct OV,UV,OC,OT:
%     should this flag be asserted at least once?), SOC_final_expected
%     (% or NaN), description.
%
%   Scenarios 1-10 run with the simulated contactor DISABLED so that every
%   fault flag stays observable for the whole run. Scenario 11 enables it.

    if nargin < 2, P = bms_params(); end
    nan_ = NaN;
    prot = false;
    Tamb = @(t) 25 + 0 * t;
    T0   = 25;
    SOCf = nan_;

    switch id
        case 1
            sc.name = 'Normal discharge';
            t_end = 1800; SOC0 = 80; I = @(t) 20 + 0 * t;
            exp_ = [0 0 0 0]; SOCf = 60;
            sc.description = '20 A constant discharge for 30 min from 80 %.';
        case 2
            sc.name = 'Battery charging';
            t_end = 3600; SOC0 = 30; I = @(t) -25 + 0 * t;
            exp_ = [0 0 0 0]; SOCf = 80;
            sc.description = '25 A constant charge (negative current) for 1 h from 30 %.';
        case 3
            sc.name = 'Variable load current';
            t_end = 3600; SOC0 = 90;
            tp = [0 300 600 900 1200 1500 1800 2100 2400 3000 3600];
            Ip = [0  40  70  -30   20   60  -20   45   10   30    0];
            I = @(t) interp1(tp, Ip, t, 'linear');
            exp_ = [0 0 0 0];
            sc.description = 'Drive-cycle-like profile with regenerative (negative) segments, peak 70 A.';
        case 4
            sc.name = 'Overvoltage';
            t_end = 600; SOC0 = 85; I = @(t) -40 + 0 * t;
            exp_ = [1 0 0 0];
            sc.description = '40 A charge from 85 %: terminal voltage rises above 54.6 V.';
        case 5
            sc.name = 'Undervoltage';
            t_end = 600; SOC0 = 12; I = @(t) 60 + 0 * t;
            exp_ = [0 1 0 0];
            sc.description = '60 A discharge from 12 %: voltage falls below 39 V near empty.';
        case 6
            sc.name = 'Overcurrent (discharge)';
            t_end = 600; SOC0 = 70; I = @(t) 20 + 100 * (t >= 300 & t < 400);
            exp_ = [0 0 1 0];
            sc.description = '20 A load with a 120 A pulse from t = 300 s to 400 s.';
        case 7
            sc.name = 'Excessive charging current';
            t_end = 600; SOC0 = 40; I = @(t) -20 - 100 * (t >= 300 & t < 400);
            exp_ = [0 0 1 0];
            sc.description = '20 A charge with a -120 A pulse from t = 300 s to 400 s.';
        case 8
            sc.name = 'Overtemperature';
            t_end = 1800; SOC0 = 95; T0 = 50; Tamb = @(t) 40 + 0 * t;
            I = @(t) 80 + 0 * t;
            exp_ = [0 0 0 1];
            sc.description = 'Hot start (50 degC), 40 degC ambient, 80 A discharge: I^2*R0 heating.';
        case 9
            sc.name = 'Multiple simultaneous faults';
            t_end = 300; SOC0 = 95; T0 = 58; Tamb = @(t) 45 + 0 * t;
            I = @(t) -130 + 0 * t;
            exp_ = [1 0 1 1];
            sc.description = '130 A charge at 95 % SOC, hot pack: OV + OC at once, OT after about 50 s.';
        case 10
            sc.name = 'SOC approaching lower limit';
            t_end = 1200; SOC0 = 15; I = @(t) 30 + 0 * t;
            exp_ = [0 1 0 0]; SOCf = 0;
            sc.description = '30 A discharge from 15 %: SOC saturates at 0 % near t = 900 s, UV near the end.';
        case 11
            sc.name = 'Protection demo (contactor trip)';
            t_end = 600; SOC0 = 70; I = @(t) 20 + 100 * (t >= 300 & t < 400);
            exp_ = [0 0 1 0]; prot = true;
            sc.description = 'Same as scenario 6 but the simulated contactor opens and latches.';
        otherwise
            error('bms_scenarios:badId', 'Scenario id must be 1..11.');
    end

    t = (0:P.dt:t_end).';
    sc.id         = id;
    sc.t          = t;
    sc.I_cmd      = I(t);
    sc.T_amb      = Tamb(t);
    sc.SOC_init   = SOC0;
    sc.T_init     = T0;
    sc.protection = prot;
    sc.expect     = struct('OV', exp_(1) == 1, 'UV', exp_(2) == 1, ...
                           'OC', exp_(3) == 1, 'OT', exp_(4) == 1);
    sc.SOC_final_expected = SOCf;
end

### 3.6 simulate_bms.m

function R = simulate_bms(P, sc)
%SIMULATE_BMS  Time-stepping simulation: battery plant + BMS + fault logic.
%
%   R = simulate_bms(P, sc)
%     P   parameter struct from bms_params()
%     sc  scenario struct from bms_scenarios()
%
%   Per sample k:
%     1. The commanded current passes through the simulated contactor.
%     2. The "true" battery SOC, voltage and temperature are updated.
%     3. The BMS "measures" current (with optional offset) and estimates SOC
%        by Coulomb counting from the MEASURED current.
%     4. Fault flags are evaluated; if protection is enabled, a fault latches
%        the simulated contactor open for the NEXT sample.
%
%   The simulated contactor is a software signal only. A real BMS drives a
%   hardware contactor / MOSFET driver with redundancy and diagnostics.

    t  = sc.t(:);   N = numel(t);   dt = P.dt;

    SOC_true = zeros(N,1);  SOC_est = zeros(N,1);
    V = zeros(N,1);  I_true = zeros(N,1);  I_meas = zeros(N,1);  T = zeros(N,1);
    OV = false(N,1); UV = false(N,1); OC = false(N,1); OT = false(N,1);
    ANY = false(N,1);  closed_log = true(N,1);

    SOC_true(1) = sc.SOC_init;
    if isempty(P.SOC_est_init), SOC_est(1) = sc.SOC_init;
    else,                       SOC_est(1) = P.SOC_est_init; end
    T(1) = sc.T_init;

    closed  = true;     % contactor state (true = closed, current can flow)
    latched = false;    % fault latch

    for k = 1:N
        closed_log(k) = closed;
        I_true(k) = sc.I_cmd(k) * closed;

        if k > 1
            SOC_true(k) = coulomb_counting(I_true(k), dt, P.Q_Ah, SOC_true(k-1));
            heat = I_true(k)^2 * P.R0;                    % W  (I^2 R)
            loss = (T(k-1) - sc.T_amb(k)) / P.Rth;        % W  to ambient
            T(k) = T(k-1) + dt / P.Cth * (heat - loss);
        end

        V(k) = battery_voltage_model(SOC_true(k), I_true(k), P.R0, ...
                                     P.SOC_bp, P.OCV_pack);

        I_meas(k) = I_true(k) + P.I_offset;               % sensor error
        if k > 1
            SOC_est(k) = coulomb_counting(I_meas(k), dt, P.Q_Ah, SOC_est(k-1));
        end

        [OV(k), UV(k), OC(k), OT(k), ANY(k)] = bms_fault_detection( ...
            V(k), I_meas(k), T(k), P.V_max, P.V_min, P.I_max, P.T_max);

        if sc.protection && ANY(k), latched = true; end
        closed = ~(sc.protection && latched);
    end

    R.t = t;  R.SOC_true = SOC_true;  R.SOC_est = SOC_est;
    R.V = V;  R.I = I_true;  R.I_meas = I_meas;  R.T = T;
    R.OV = OV; R.UV = UV; R.OC = OC; R.OT = OT; R.any_fault = ANY;
    R.contactor_closed = closed_log;
    R.name = sc.name;  R.sc = sc;  R.P = P;
end

### 3.7 bms_stageA_signals.m

function S = bms_stageA_signals(P)
%BMS_STAGEA_SIGNALS  Stage A: synthetic voltage/current/temperature signals.
%
%   Each fault is created in its own time window, so you can see every flag
%   rise and fall independently. Used by EV_BMS_Main.m and by the Simulink
%   model (From Workspace blocks) when stage_select = 0.
%
%   Windows (seconds):  0-99 normal | 100-149 OV | 200-249 UV |
%     300-349 OC discharge | 400-449 OC charge | 500-549 OT | 560-599 all four-ish
%   (the last window raises OV + OC + OT together).

    if nargin < 1, P = bms_params(); end
    t = (0:P.dt:600).';
    V = 48 + 0 * t;   I = 20 + 0 * t;   T = 30 + 0 * t;

    V(t >= 100 & t < 150) = P.V_max + 1.5;        % overvoltage
    V(t >= 200 & t < 250) = P.V_min - 2.0;        % undervoltage
    I(t >= 300 & t < 350) = P.I_max + 20;         % discharge overcurrent
    I(t >= 400 & t < 450) = -(P.I_max + 20);      % charge overcurrent
    T(t >= 500 & t < 550) = P.T_max + 5;          % overtemperature
    V(t >= 560 & t < 600) = P.V_max + 1.5;        % multiple faults
    I(t >= 560 & t < 600) = P.I_max + 20;
    T(t >= 560 & t < 600) = P.T_max + 5;

    S.t = t;  S.V = V;  S.I = I;  S.T = T;
    S.expected_windows = struct( ...
        'OV', [100 150; 560 600], 'UV', [200 250], ...
        'OC', [300 350; 400 450; 560 600], 'OT', [500 550; 560 600]);
end

### 3.8 plot_bms_results.m

function fig = plot_bms_results(R, fig_title)
%PLOT_BMS_RESULTS  Six-panel result figure: SOC, V, I, T, fault flags, combined.
%   R is the struct returned by simulate_bms (or built by EV_BMS_StageA).

    if nargin < 2, fig_title = R.name; end
    P = R.P;  t = R.t / 60;                 % plot time in minutes
    fig = figure('Name', fig_title, 'NumberTitle', 'off', 'Position', [60 60 1100 800]);

    subplot(3,2,1);
    plot(t, R.SOC_true, 'b', t, R.SOC_est, 'r--', 'LineWidth', 1.4);
    ylabel('SOC (%)'); ylim([-5 105]); grid on; legend('True', 'Coulomb counting', 'Location', 'best');
    title('State of charge');

    subplot(3,2,2);
    plot(t, R.V, 'b', 'LineWidth', 1.4); hold on;
    plot([t(1) t(end)], [P.V_max P.V_max], 'r--', [t(1) t(end)], [P.V_min P.V_min], 'm--');
    ylabel('Voltage (V)'); grid on; legend('V', 'V_{max}', 'V_{min}', 'Location', 'best');
    title('Terminal voltage');

    subplot(3,2,3);
    plot(t, R.I_meas, 'b', 'LineWidth', 1.4); hold on;
    plot([t(1) t(end)], [P.I_max P.I_max], 'r--', [t(1) t(end)], [-P.I_max -P.I_max], 'r--');
    ylabel('Current (A)'); grid on; title('Current (+ discharge, - charge)');

    subplot(3,2,4);
    plot(t, R.T, 'b', 'LineWidth', 1.4); hold on;
    plot([t(1) t(end)], [P.T_max P.T_max], 'r--');
    ylabel('Temperature (^{\circ}C)'); grid on; title('Pack temperature');

    subplot(3,2,5);
    stairs(t, double(R.OV) + 3.0, 'r', 'LineWidth', 1.2); hold on;
    stairs(t, double(R.UV) + 2.0, 'm', 'LineWidth', 1.2);
    stairs(t, double(R.OC) + 1.0, 'k', 'LineWidth', 1.2);
    stairs(t, double(R.OT) + 0.0, 'Color', [0.9 0.5 0], 'LineWidth', 1.2);
    set(gca, 'YTick', [0.5 1.5 2.5 3.5], 'YTickLabel', {'OT', 'OC', 'UV', 'OV'});
    ylim([-0.2 4.4]); xlabel('Time (min)'); grid on; title('Individual fault flags');

    subplot(3,2,6);
    stairs(t, double(R.any_fault), 'r', 'LineWidth', 1.4); hold on;
    stairs(t, double(R.contactor_closed) * 0.9, 'g--', 'LineWidth', 1.2);
    ylim([-0.1 1.2]); xlabel('Time (min)'); grid on;
    legend('Combined fault', 'Contactor closed', 'Location', 'best');
    title('Combined fault and simulated contactor');
end

### 3.9 EV_BMS_Main.m

%% EV_BMS_Main.m
%  EV Battery Management System with SOC estimation (Coulomb counting)
%  Main script: Stage A (synthetic signals) + Stage B (battery model) scenarios.
%
%  Run:  >> EV_BMS_Main
%  Needs only base MATLAB (no toolboxes). All files must be in the same folder.
%  Sign convention: current > 0 = discharge, current < 0 = charge.

clear; clc; close all;

%% ----------------------------- user options -------------------------------
SHOW_PLOTS   = true;        % false for a fast, text-only run
PLOT_IDS     = [3 9];       % scenarios to plot in detail (1..11)
SAVE_FIGURES = false;       % true: save PNGs into ./results
SOC_TOL      = 0.05;        % % tolerance for the analytic SOC check

P = bms_params();           % <- change battery/threshold values in bms_params.m

fprintf('=== EV BMS: %g V, %g Ah pack | V %g..%g V | |I| <= %g A | T <= %g C ===\n', ...
        P.V_nom, P.Q_Ah, P.V_min, P.V_max, P.I_max, P.T_max);
fprintf('Thresholds are ILLUSTRATIVE; use the cell datasheet for a real pack.\n\n');

%% ------------------------- STAGE A: synthetic signals ---------------------
fprintf('--- Stage A: synthetic V/I/T signals ---\n');
S = bms_stageA_signals(P);
[A.OV, A.UV, A.OC, A.OT, A.any_fault] = bms_fault_detection( ...
    S.V, S.I, S.T, P.V_max, P.V_min, P.I_max, P.T_max);
A.SOC_est = coulomb_counting(S.I, P.dt, P.Q_Ah, P.SOC_init);
A.SOC_true = A.SOC_est;  A.t = S.t;  A.V = S.V;  A.I_meas = S.I;  A.T = S.T;
A.contactor_closed = true(size(S.t));  A.name = 'Stage A synthetic signals';  A.P = P;

names = {'OV', 'UV', 'OC', 'OT'};
stageA_ok = true;
for n = 1:4
    w = S.expected_windows.(names{n});
    expected = false(size(S.t));
    for r = 1:size(w, 1), expected = expected | (S.t >= w(r,1) & S.t < w(r,2)); end
    ok = isequal(A.(names{n}), expected);
    stageA_ok = stageA_ok && ok;
    fprintf('  %-2s flag matches expected windows : %s\n', names{n}, passfail(ok));
end
fprintf('  Stage A overall: %s\n\n', passfail(stageA_ok));
if SHOW_PLOTS, plot_bms_results(A, 'Stage A: synthetic signals'); end

%% ------------------------- STAGE B: scenarios 1..11 -----------------------
fprintf('--- Stage B: battery model scenarios ---\n');
hdr = sprintf('%-3s %-32s %-4s %-4s %-4s %-4s %8s %7s %7s %7s  %s', ...
    '#', 'Scenario', 'OV', 'UV', 'OC', 'OT', 'SOC_end', 'Vmin', 'Vmax', 'Tmax', 'Result');
fprintf('%s\n%s\n', hdr, repmat('-', 1, numel(hdr)));

all_ok = stageA_ok;
results = cell(1, 11);
for id = 1:11
    sc = bms_scenarios(id, P);
    R  = simulate_bms(P, sc);
    results{id} = R;

    got = [any(R.OV) any(R.UV) any(R.OC) any(R.OT)];
    exp_ = [sc.expect.OV sc.expect.UV sc.expect.OC sc.expect.OT];
    ok = isequal(got, exp_);
    if ~isnan(sc.SOC_final_expected)
        ok = ok && abs(R.SOC_true(end) - sc.SOC_final_expected) <= SOC_TOL ...
                && abs(R.SOC_est(end)  - sc.SOC_final_expected) <= SOC_TOL;
    end
    if sc.protection      % contactor must have opened and stayed open
        ok = ok && ~R.contactor_closed(end) && R.I(end) == 0;
    end
    all_ok = all_ok && ok;

    fprintf('%-3d %-32s %-4d %-4d %-4d %-4d %7.2f%% %7.2f %7.2f %7.2f  %s\n', ...
        id, sc.name, got(1), got(2), got(3), got(4), R.SOC_true(end), ...
        min(R.V), max(R.V), max(R.T), passfail(ok));

    if SHOW_PLOTS && any(id == PLOT_IDS)
        f = plot_bms_results(R, sprintf('Scenario %d: %s', id, sc.name));
        if SAVE_FIGURES
            if ~exist('results', 'dir'), mkdir('results'); end
            saveas(f, fullfile('results', sprintf('scenario_%02d.png', id)));
        end
    end
end

%% --------------------- first-trip times (useful for the viva) -------------
fprintf('\nFirst assertion time of each fault flag (s):\n');
for id = 4:11
    R = results{id};
    fprintf('  Scenario %2d (%s):', id, R.name);
    for n = 1:4
        idx = find(R.(names{n}), 1, 'first');
        if isempty(idx), fprintf('  %s=--', names{n});
        else,            fprintf('  %s=%g', names{n}, R.t(idx)); end
    end
    fprintf('\n');
end

%% ---------------- manual SOC check (matches the guide, section 10) --------
R1 = results{1};
manual = P.SOC_init - 20 * 1800 / (3600 * P.Q_Ah) * 100;
fprintf('\nManual check, scenario 1: 80 - (20 A x 1800 s)/(3600 x 50 Ah) x 100 = %.2f %%\n', manual);
fprintf('Code result: %.2f %%\n', R1.SOC_est(end));

%% ------------- effect of a 2 A current-sensor offset (drift demo) ---------
P2 = P;  P2.I_offset = 2;                       % BMS sees 2 A too much
R_off = simulate_bms(P2, bms_scenarios(1, P2));
fprintf('\nSensor-offset demo (scenario 1, +2 A offset): true SOC %.2f %%, estimated %.2f %%, error %.2f %%\n', ...
        R_off.SOC_true(end), R_off.SOC_est(end), R_off.SOC_est(end) - R_off.SOC_true(end));

fprintf('\n=== OVERALL: %s ===\n', passfail(all_ok));

function s = passfail(ok)
    if ok, s = 'PASS'; else, s = 'FAIL'; end
end

### 3.10 Optional advanced module: EKF

Model. Two states, x = [SOC; V1], with a first-order RC (Thevenin) cell:

- Prediction: SOC(k) = SOC(k-1) − I·dt/(3600 Q), V1(k) = a·V1(k-1) + R1(1−a)·I, a = e^(−dt/(R1 C1)), P⁻ = F P Fᵀ + Qn.

- Measurement: V = OCV(SOC) − V1 − R0·I.

- Update: H = [dOCV/dSOC, −1], K = P⁻Hᵀ/(H P⁻ Hᵀ + Rn), x = x⁻ + K (V_meas − V_model), P = (I − KH)P⁻.

What a meaningful EKF needs: a measured OCV–SOC table, R0/R1/C1 identified from pulse tests at several temperatures, sensor noise statistics, tuned Qn/Rn, and validation against reference SOC. The demo uses a synthetic cell that is exactly the EKF’s own model, so the good result there is a best case and not evidence of accuracy.

function [SOC_ekf, V1_ekf, P_trace] = ekf_soc_estimator(V_meas, I_meas, dt, E)
%EKF_SOC_ESTIMATOR  ADVANCED, OPTIONAL: Extended Kalman Filter SOC estimator.
%
%   Battery model (1-RC Thevenin):
%       states   x = [SOC (fraction 0..1) ; V1 (V, RC polarisation voltage)]
%       SOC(k) = SOC(k-1) - I(k)*dt/(3600*Q)
%       V1(k)  = a*V1(k-1) + R1*(1-a)*I(k),   a = exp(-dt/(R1*C1))
%       V(k)   = OCV(SOC(k)) - V1(k) - R0*I(k)           (measurement)
%
%   PREDICT :  x- = f(x, I),   P- = F*P*F' + Qn
%   UPDATE  :  H  = [dOCV/dSOC, -1]
%              K  = P- H' / (H P- H' + Rn)
%              x  = x- + K*(V_meas - V_model),  P = (I - K H) P-
%
%   Inputs  V_meas, I_meas : measured terminal voltage (V), current (A, + = discharge)
%           dt             : sampling time (s)
%           E              : struct with fields
%               Q_Ah, R0, R1, C1                    cell/pack parameters
%               SOC_bp, OCV_pack                    OCV-SOC table (SOC in %)
%               SOC0 (%)                            initial guess
%               P0 (2x2)  initial covariance,  Qn (2x2) process noise,
%               Rn (scalar) measurement-noise variance (V^2)
%   Outputs SOC_ekf in %, V1_ekf in V, P_trace = trace of P at each step.
%
%   WHAT YOU NEED FOR A MEANINGFUL EKF: (1) an OCV-SOC table measured from
%   your cell, (2) R0, R1, C1 identified from pulse tests (or HPPC tests) at
%   the relevant temperatures, (3) sensor noise statistics, (4) tuning of
%   Qn / Rn, (5) VALIDATION against reference SOC (e.g. a lab cycler's
%   Ah-count with a precise current sensor). Without (1)-(5) this code
%   only demonstrates the algorithm; it is NOT a validated estimator.

    V_meas = V_meas(:);  I_meas = I_meas(:);  N = numel(V_meas);
    if numel(I_meas) ~= N, error('ekf_soc_estimator:size', 'V and I must match in length.'); end

    a = exp(-dt / (E.R1 * E.C1));
    x = [E.SOC0 / 100; 0];
    Pc = E.P0;
    F = [1 0; 0 a];

    SOC_ekf = zeros(N,1);  V1_ekf = zeros(N,1);  P_trace = zeros(N,1);
    for k = 1:N
        if k > 1                                             % ---- predict
            x = [x(1) - I_meas(k) * dt / (3600 * E.Q_Ah); ...
                 a * x(2) + E.R1 * (1 - a) * I_meas(k)];
            Pc = F * Pc * F' + E.Qn;
        end
        soc_pct = min(max(x(1) * 100, 0), 100);              % ---- update
        ocv   = interp1(E.SOC_bp, E.OCV_pack, soc_pct, 'linear');
        s_hi  = min(soc_pct + 0.5, 100);  s_lo = max(soc_pct - 0.5, 0);
        dOCV  = (interp1(E.SOC_bp, E.OCV_pack, s_hi) - ...
                 interp1(E.SOC_bp, E.OCV_pack, s_lo)) / (s_hi - s_lo);   % V per %
        H     = [dOCV * 100, -1];                            % V per unit fraction
        V_hat = ocv - x(2) - E.R0 * I_meas(k);
        Sk    = H * Pc * H' + E.Rn;
        K     = Pc * H' / Sk;
        x     = x + K * (V_meas(k) - V_hat);
        Pc    = (eye(2) - K * H) * Pc;
        x(1)  = min(max(x(1), 0), 1);

        SOC_ekf(k) = x(1) * 100;  V1_ekf(k) = x(2);  P_trace(k) = trace(Pc);
    end
end

%% EV_BMS_EKF_demo.m   (ADVANCED, OPTIONAL)
%  Compares Coulomb counting vs an EKF when the estimator starts from a WRONG
%  initial SOC (50 % instead of 80 %).
%
%  HONEST CAVEAT: the "true" battery below is a synthetic 1-RC model that is
%  EXACTLY the model the EKF assumes, with simulated noise. Good results here
%  show the algorithm works in a best case; they say nothing about accuracy
%  on a real cell. Validate against reference SOC data before claiming accuracy.

clear; clc; close all;
P = bms_params();
dt = 1;  rng(1);                                   % repeatable noise

% --- synthetic "real" cell with RC dynamics (illustrative values) -----------
R1 = 0.015;  C1 = 2000;  a = exp(-dt / (R1 * C1));
tp = [0 300 600 900 1200 1500 1800 2100 2400 3000 3600];
Ip = [0  40  70  -30   20   60  -20   45   10   30    0];
t = (0:dt:3600).';   I = interp1(tp, Ip, t);

SOC_true = zeros(size(t));  V1 = zeros(size(t));  V = zeros(size(t));
SOC_true(1) = 80;
for k = 1:numel(t)
    if k > 1
        SOC_true(k) = SOC_true(k-1) - I(k) * dt / (3600 * P.Q_Ah) * 100;
        V1(k) = a * V1(k-1) + R1 * (1 - a) * I(k);
    end
    V(k) = battery_voltage_model(SOC_true(k), 0, 0, P.SOC_bp, P.OCV_pack) - V1(k) - P.R0 * I(k);
end
V_meas = V + 0.02 * randn(size(V));                % 20 mV voltage noise
I_meas = I + 0.5  * randn(size(I));                % 0.5 A current noise

% --- Coulomb counting with the wrong start ---------------------------------
SOC_cc = coulomb_counting(I_meas, dt, P.Q_Ah, 50);

% --- EKF with the same wrong start but large initial uncertainty -----------
E = struct('Q_Ah', P.Q_Ah, 'R0', P.R0, 'R1', R1, 'C1', C1, ...
           'SOC_bp', P.SOC_bp, 'OCV_pack', P.OCV_pack, 'SOC0', 50, ...
           'P0', diag([0.3^2, 0.05^2]), 'Qn', diag([1e-9, 1e-6]), 'Rn', 0.05^2);
[SOC_ekf, ~, P_trace] = ekf_soc_estimator(V_meas, I_meas, dt, E);

% --- results ----------------------------------------------------------------
half = t > 1800;
fprintf('RMS SOC error, last 30 min:  Coulomb counting %.2f %%   EKF %.2f %%\n', ...
    sqrt(mean((SOC_cc(half) - SOC_true(half)).^2)), ...
    sqrt(mean((SOC_ekf(half) - SOC_true(half)).^2)));

figure('Name', 'EKF vs Coulomb counting');
subplot(2,1,1);
plot(t/60, SOC_true, 'k', t/60, SOC_cc, 'r--', t/60, SOC_ekf, 'b', 'LineWidth', 1.3);
grid on; ylabel('SOC (%)'); legend('True (synthetic)', 'Coulomb counting, wrong start', 'EKF, wrong start');
title('Recovery from a wrong initial SOC (model-matched synthetic test)');
subplot(2,1,2);
plot(t/60, SOC_ekf - SOC_true, 'b', 'LineWidth', 1.2); grid on;
xlabel('Time (min)'); ylabel('EKF error (%)');

## 4. Simulink model block diagram

Top level of EV_BMS_SOC.slx (signal flow):

ILoad_Profile --> [Protection_Control] --I_actual--> [Battery_Model] <-- Tamb_Profile
   (From WS)         ^   (latch)                      |V |I |T |SOC_true
                     |Fault_any                       v  v  v
                     |                       [Switch_V][Switch_I][Switch_T] <-- V/I/T_Synthetic (From WS)
                     |                                \   |   /     ^
                     |                                 v  v  v      Stage_Select (Constant)
                     |                          [Battery_Monitoring]  (+ I_offset Sum block)
                     |                           |V_meas |I_meas |T_meas |SOC_mon
                     |                           |       |       |       ^
                     |                           |       +-------|-----> [SOC_Estimator] (Gain -> Integrator -> Saturation)
                     |                           v       v       v
                     +---------------------- [Fault_Detection] --> OV UV OC OT ANY
                                                 |
 [Visualization]  <-- SOC_true, SOC_est, V, I, T, OV, UV, OC, OT, ANY, Contactor
 [To Workspace log_* blocks] <-- the same signals

Inside the subsystems:

Battery_Model
  I_load --> Gain_SOC(k_soc) --> Int_SOC(IC=SOC_init, limit 0..100) --> OCV_Lookup --+--> Sum_V(+ -) --> V_batt
  SOC0_const ---------------------> Int_SOC (x0 port)                  Gain_R0(R0) --+
  I_load --> Prod_I2 --> Gain_Heat(R0) --> Sum_Net(+ -) --> Gain_Cth_inv --> Int_T(IC=T_init) --> T_batt
  Int_T, T_amb --> Sum_dT(+ -) --> Gain_Rth_inv --------^ (loss)       (Int_T also feeds back to Sum_dT)

Fault_Detection
  V --> Rel_OV(>, Vmax)      --> OV  --+
  V --> Rel_UV(<, Vmin)      --> UV  --+--> OR_Any (4 inputs) --> ANY
  I --> Abs_I --> Rel_OC(>, Imax) --> OC --+
  T --> Rel_OT(>, Tmax)      --> OT  --+

Protection_Control
  Fault_any --> OR_Latch --> Latch_Memory (Unit Delay) --+--> OR_Latch (2nd input)
                                                         +--> AND_Enable (with Prot_enable) --> NOT_Trip
  --> To_Double --> Gate_I (Product with I_cmd) --> I_actual

## 5. Exact block parameters and connections

create_EV_BMS_model.m sets every one of these. The table is for understanding the model, or for building it by hand.

Model settings: Solver ode1 (Euler), fixed-step, step Ts (= 1 s), stop time Tstop, InitFcn bms_simulink_init;.

| Subsystem | Block (library) | Key parameters |
| --- | --- | --- |
| Battery_Model | Gain_SOC (Gain) | Gain k_soc = −100/(3600·Q_Ah) |
|  | Int_SOC (Integrator) | Initial condition source external, limit output on, upper 100, lower 0 |
|  | SOC0_const (Constant) | SOC_init → Int_SOC port 2 |
|  | OCV_Lookup (1-D Lookup Table) | Breakpoints SOC_bp, Table OCV_pack, extrapolation Clip |
|  | Gain_R0 (Gain), Sum_V (Sum) | Gain R0; Sum signs +- (OCV − I·R0) |
|  | Prod_I2 (Product), Gain_Heat | Both inputs = I; Gain R0 (heat = I²R0) |
|  | Sum_dT (+-), Gain_Rth_inv | (T − T_amb), Gain inv_Rth |
|  | Sum_Net (+-), Gain_Cth_inv, Int_T | heat − loss; Gain inv_Cth; IC T_init |
| SOC_Estimator | Gain_SOC, Int_SOC, SOC_est0, Sat_SOC | Gain k_soc; external IC SOC_est_init; limits 0/100 |
| Battery_Monitoring | Sum_I_sensor (Sum ++) + I_offset_const | Current-sensor offset I_offset (A) |
| Fault_Detection | Rel_OV >, Rel_UV <, Rel_OC >, Rel_OT > | Compared to constants V_max, V_min, I_max, T_max |
|  | Abs_I (Abs) |  |
|  | OR_Any (Logical Operator) | Operator OR, 4 inputs |
| Protection_Control | OR_Latch, Latch_Memory | OR 2 inputs; Unit Delay, IC 0, sample time Ts |
|  | Prot_enable (Constant), AND_Enable, NOT_Trip | boolean constant logical(protection_enable); AND 2; NOT |
|  | To_Double, Gate_I | Data type double; Product (I_cmd × contactor) |
| Visualization | 5 Scopes, 3 Displays | Scope_SOC (2 ports), Scope_Faults (5 ports), Display_SOC, Display_Fault_Any, Display_Contactor |

Why a one-sample Unit Delay in the latch? The loop fault → contactor → current → voltage → fault would otherwise be an algebraic loop. The delay mirrors real hardware, where the contactor opens a moment after the fault is detected.

Integrator wind-up. With the Integrator’s own 0–100 limit turned on, the integrator stops integrating at the limit. A separate Saturation after an unlimited integrator would let the hidden state keep growing past 100 %, so SOC would stay stuck at the limit after the current reverses.

All blocks are core Simulink. No other toolbox is needed. If your release names a parameter differently, see section 12.

## 6. MATLAB script that generates the SLX model

create_EV_BMS_model.m is a function file (call it by typing its name). It builds the whole model, saves EV_BMS_SOC.slx, runs a compile check (SimulationCommand update) and a short test run, and prints any error.

### 6.1 bms_simulink_init.m

% bms_simulink_init.m   (script, runs in the BASE workspace)
%  Creates every workspace variable that EV_BMS_SOC.slx uses in its block
%  parameters. The model calls this automatically (InitFcn) before each run.
%
%  Choose before running:
%     scenario_id  = 1..11   (default 1)   which bms_scenarios() case drives the load
%     stage_select = 1       Stage B: battery model produces V, I, T   (default)
%                  = 0       Stage A: synthetic V, I, T signals are used instead

if ~exist('scenario_id', 'var'),  scenario_id  = 1; end %#ok<*NASGU>
if ~exist('stage_select', 'var'), stage_select = 1; end

P_  = bms_params();
sc_ = bms_scenarios(scenario_id, P_);
SA_ = bms_stageA_signals(P_);

% ---- battery / protection parameters (names used in block dialogs) ---------
Q_Ah     = P_.Q_Ah;
SOC_init = sc_.SOC_init;
if isempty(P_.SOC_est_init), SOC_est_init = sc_.SOC_init; else, SOC_est_init = P_.SOC_est_init; end
T_init   = sc_.T_init;
R0       = P_.R0;
inv_Rth  = 1 / P_.Rth;                 % 1/(K/W)
inv_Cth  = 1 / P_.Cth;                 % 1/(J/K)
V_max    = P_.V_max;   V_min = P_.V_min;   I_max = P_.I_max;   T_max = P_.T_max;
I_offset = P_.I_offset;
SOC_bp   = P_.SOC_bp;  OCV_pack = P_.OCV_pack;
protection_enable = double(sc_.protection);

% ---- SOC integrator gain: % per (A*s)  = -100 / (3600 * Q_Ah) ---------------
k_soc    = -100 / (3600 * Q_Ah);       % e.g. 50 Ah -> -5.5556e-4

% ---- time base ---------------------------------------------------------------
Ts = P_.dt;
if stage_select == 1, Tstop = sc_.t(end); else, Tstop = SA_.t(end); end

% ---- From Workspace data: [time, value] matrices ---------------------------
I_load_data = [sc_.t, sc_.I_cmd];
Tamb_data   = [sc_.t, sc_.T_amb];
V_sig_data  = [SA_.t, SA_.V];
I_sig_data  = [SA_.t, SA_.I];
T_sig_data  = [SA_.t, SA_.T];

### 6.2 create_EV_BMS_model.m

function create_EV_BMS_model(run_test)
%CREATE_EV_BMS_MODEL  Programmatically builds and saves EV_BMS_SOC.slx.
%
%   >> create_EV_BMS_model          % build, save, compile-check, short test run
%   >> create_EV_BMS_model(false)   % build and save only
%
%   REQUIRES: MATLAB + Simulink only (all blocks are core Simulink; no
%   Simscape, Stateflow or other toolbox). Keep all project files in one folder.
%
%   STATUS: this script was NOT run inside MATLAB/Simulink by its author (no
%   MATLAB was available). Its block list and connections were checked by a
%   separate structural checker, but you must run it once to confirm it works
%   on your release. See "Troubleshooting" in the guide if a line errors.
%
%   Model structure (top level):
%     ILoad_Profile -> Protection_Control -> Battery_Model -> [Stage switches]
%        -> Battery_Monitoring -> SOC_Estimator
%                              -> Fault_Detection -> Protection_Control (loop)
%        -> Visualization (scopes, displays)  and  To Workspace logging

    if nargin < 1, run_test = true; end
    mdl = 'EV_BMS_SOC';

    % ---- make sure workspace variables exist, start from a clean model ------
    evalin('base', 'bms_simulink_init;');
    if bdIsLoaded(mdl), close_system(mdl, 0); end
    if exist([mdl '.slx'], 'file'), delete([mdl '.slx']); end
    new_system(mdl);
    open_system(mdl);

    % ---- solver configuration: fixed-step Euler, 1 s (= P.dt) --------------
    set_param(mdl, 'SolverType', 'Fixed-step');
    set_param(mdl, 'Solver', 'ode1');
    set_param(mdl, 'FixedStep', 'Ts');
    set_param(mdl, 'StopTime', 'Tstop');
    set_param(mdl, 'InitFcn', 'bms_simulink_init;');
    set_param(mdl, 'SaveTime', 'on');
    set_param(mdl, 'ReturnWorkspaceOutputs', 'on');

    %% =================== SUBSYSTEM 1: Battery_Model ==========================
    % Inputs : I_load (A, + discharge), T_amb (degC)
    % Outputs: V_batt (V), I_batt (A), T_batt (degC), SOC_true (%)
    % V = OCV(SOC) - I*R0 ;  C*dT/dt = I^2*R0 - (T - Tamb)/Rth
    sys = mdl;
    blk(sys, 'Battery_Model', 'built-in/Subsystem', [420 60 560 200]);
    sys = [mdl '/Battery_Model'];
    blk(sys, 'I_load',        'simulink/Sources/In1',                [20 40 50 54],    'Port', '1');
    blk(sys, 'T_amb',         'simulink/Sources/In1',                [20 300 50 314],  'Port', '2');
    blk(sys, 'Gain_SOC',      'simulink/Math Operations/Gain',       [110 20 170 50],  'Gain', 'k_soc');
    blk(sys, 'SOC0_const',    'simulink/Sources/Constant',           [110 80 170 110], 'Value', 'SOC_init');
    blk(sys, 'Int_SOC',       'simulink/Continuous/Integrator',      [220 30 260 100], 'InitialConditionSource', 'external', 'LimitOutput', 'on', 'UpperSaturationLimit', '100', 'LowerSaturationLimit', '0');
    blk(sys, 'OCV_Lookup',    'simulink/Lookup Tables/1-D Lookup Table', [310 30 380 60], 'BreakpointsForDimension1', 'SOC_bp', 'Table', 'OCV_pack');
    blk(sys, 'Gain_R0',       'simulink/Math Operations/Gain',       [110 140 170 170], 'Gain', 'R0');
    blk(sys, 'Sum_V',         'simulink/Math Operations/Sum',        [430 40 460 80],  'Inputs', '+-');
    blk(sys, 'Prod_I2',       'simulink/Math Operations/Product',    [110 220 140 260], 'Inputs', '2');
    blk(sys, 'Gain_Heat',     'simulink/Math Operations/Gain',       [180 225 240 255], 'Gain', 'R0');
    blk(sys, 'Sum_dT',        'simulink/Math Operations/Sum',        [180 290 210 330], 'Inputs', '+-');
    blk(sys, 'Gain_Rth_inv',  'simulink/Math Operations/Gain',       [240 295 300 325], 'Gain', 'inv_Rth');
    blk(sys, 'Sum_Net',       'simulink/Math Operations/Sum',        [340 235 370 275], 'Inputs', '+-');
    blk(sys, 'Gain_Cth_inv',  'simulink/Math Operations/Gain',       [400 240 460 270], 'Gain', 'inv_Cth');
    blk(sys, 'Int_T',         'simulink/Continuous/Integrator',      [500 235 540 275], 'InitialCondition', 'T_init');
    blk(sys, 'V_batt',        'simulink/Sinks/Out1',                 [600 52 630 66],  'Port', '1');
    blk(sys, 'I_batt',        'simulink/Sinks/Out1',                 [600 112 630 126], 'Port', '2');
    blk(sys, 'T_batt',        'simulink/Sinks/Out1',                 [600 245 630 259], 'Port', '3');
    blk(sys, 'SOC_true',      'simulink/Sinks/Out1',                 [600 12 630 26],  'Port', '4');
    ln(sys, 'I_load/1',       'Gain_SOC/1');
    ln(sys, 'Gain_SOC/1',     'Int_SOC/1');
    ln(sys, 'SOC0_const/1',   'Int_SOC/2');
    ln(sys, 'Int_SOC/1',      'OCV_Lookup/1');
    ln(sys, 'Int_SOC/1',      'SOC_true/1');
    ln(sys, 'I_load/1',       'Gain_R0/1');
    ln(sys, 'OCV_Lookup/1',   'Sum_V/1');
    ln(sys, 'Gain_R0/1',      'Sum_V/2');
    ln(sys, 'Sum_V/1',        'V_batt/1');
    ln(sys, 'I_load/1',       'I_batt/1');
    ln(sys, 'I_load/1',       'Prod_I2/1');
    ln(sys, 'I_load/1',       'Prod_I2/2');
    ln(sys, 'Prod_I2/1',      'Gain_Heat/1');
    ln(sys, 'Int_T/1',        'Sum_dT/1');
    ln(sys, 'T_amb/1',        'Sum_dT/2');
    ln(sys, 'Sum_dT/1',       'Gain_Rth_inv/1');
    ln(sys, 'Gain_Heat/1',    'Sum_Net/1');
    ln(sys, 'Gain_Rth_inv/1', 'Sum_Net/2');
    ln(sys, 'Sum_Net/1',      'Gain_Cth_inv/1');
    ln(sys, 'Gain_Cth_inv/1', 'Int_T/1');
    ln(sys, 'Int_T/1',        'T_batt/1');
    % Clip (do not extrapolate) outside the table; parameter name varies by release.
    try, set_param([sys '/OCV_Lookup'], 'ExtrapMethod', 'Clip'); catch, end

    %% =================== SUBSYSTEM 2: SOC_Estimator ==========================
    % SOC(t) = SOC0 + integral( -100/(3600*Q) * I dt ), limited to 0..100 %.
    % Gain = -100/(3600*Q_Ah): A*s -> Ah -> fraction of Q -> percent, minus = discharge.
    sys = mdl;
    blk(sys, 'SOC_Estimator', 'built-in/Subsystem', [980 420 1120 500]);
    sys = [mdl '/SOC_Estimator'];
    blk(sys, 'I_meas',    'simulink/Sources/In1',               [20 40 50 54],   'Port', '1');
    blk(sys, 'Gain_SOC',  'simulink/Math Operations/Gain',      [100 30 160 60], 'Gain', 'k_soc');
    blk(sys, 'SOC_est0',  'simulink/Sources/Constant',          [100 100 160 130], 'Value', 'SOC_est_init');
    blk(sys, 'Int_SOC',   'simulink/Continuous/Integrator',     [210 30 250 130], 'InitialConditionSource', 'external', 'LimitOutput', 'on', 'UpperSaturationLimit', '100', 'LowerSaturationLimit', '0');
    blk(sys, 'Sat_SOC',   'simulink/Discontinuities/Saturation', [300 50 350 80], 'UpperLimit', '100', 'LowerLimit', '0');
    blk(sys, 'SOC_est',   'simulink/Sinks/Out1',                [400 58 430 72], 'Port', '1');
    ln(sys, 'I_meas/1',   'Gain_SOC/1');
    ln(sys, 'Gain_SOC/1', 'Int_SOC/1');
    ln(sys, 'SOC_est0/1', 'Int_SOC/2');
    ln(sys, 'Int_SOC/1',  'Sat_SOC/1');
    ln(sys, 'Sat_SOC/1',  'SOC_est/1');

    %% =================== SUBSYSTEM 3: Battery_Monitoring =====================
    % Measurement layer: passes V, T, SOC and adds a configurable current-sensor
    % offset (Sum block) so you can study drift. Outputs are what the BMS "sees".
    sys = mdl;
    blk(sys, 'Battery_Monitoring', 'built-in/Subsystem', [760 200 900 340]);
    sys = [mdl '/Battery_Monitoring'];
    blk(sys, 'V',              'simulink/Sources/In1', [20 30 50 44],   'Port', '1');
    blk(sys, 'I',              'simulink/Sources/In1', [20 90 50 104],  'Port', '2');
    blk(sys, 'T',              'simulink/Sources/In1', [20 190 50 204], 'Port', '3');
    blk(sys, 'SOC',            'simulink/Sources/In1', [20 250 50 264], 'Port', '4');
    blk(sys, 'I_offset_const', 'simulink/Sources/Constant', [20 130 80 160], 'Value', 'I_offset');
    blk(sys, 'Sum_I_sensor',   'simulink/Math Operations/Sum', [140 90 170 130], 'Inputs', '++');
    blk(sys, 'V_meas',         'simulink/Sinks/Out1',  [260 30 290 44],   'Port', '1');
    blk(sys, 'I_meas',         'simulink/Sinks/Out1',  [260 103 290 117], 'Port', '2');
    blk(sys, 'T_meas',         'simulink/Sinks/Out1',  [260 190 290 204], 'Port', '3');
    blk(sys, 'SOC_mon',        'simulink/Sinks/Out1',  [260 250 290 264], 'Port', '4');
    ln(sys, 'V/1',              'V_meas/1');
    ln(sys, 'I/1',              'Sum_I_sensor/1');
    ln(sys, 'I_offset_const/1', 'Sum_I_sensor/2');
    ln(sys, 'Sum_I_sensor/1',   'I_meas/1');
    ln(sys, 'T/1',              'T_meas/1');
    ln(sys, 'SOC/1',            'SOC_mon/1');

    %% =================== SUBSYSTEM 4: Fault_Detection ========================
    % OV: V > V_max | UV: V < V_min | OC: |I| > I_max | OT: T > T_max | ANY = OR
    sys = mdl;
    blk(sys, 'Fault_Detection', 'built-in/Subsystem', [1000 200 1140 340]);
    sys = [mdl '/Fault_Detection'];
    blk(sys, 'V',        'simulink/Sources/In1', [20 30 50 44],   'Port', '1');
    blk(sys, 'I',        'simulink/Sources/In1', [20 130 50 144], 'Port', '2');
    blk(sys, 'T',        'simulink/Sources/In1', [20 230 50 244], 'Port', '3');
    blk(sys, 'Vmax_const', 'simulink/Sources/Constant', [90 5 150 25],    'Value', 'V_max');
    blk(sys, 'Vmin_const', 'simulink/Sources/Constant', [90 60 150 80],   'Value', 'V_min');
    blk(sys, 'Imax_const', 'simulink/Sources/Constant', [190 150 250 170], 'Value', 'I_max');
    blk(sys, 'Tmax_const', 'simulink/Sources/Constant', [90 255 150 275], 'Value', 'T_max');
    blk(sys, 'Abs_I',    'simulink/Math Operations/Abs', [90 125 130 150]);
    blk(sys, 'Rel_OV',   'simulink/Logic and Bit Operations/Relational Operator', [270 20 310 50],  'Operator', '>');
    blk(sys, 'Rel_UV',   'simulink/Logic and Bit Operations/Relational Operator', [270 80 310 110], 'Operator', '<');
    blk(sys, 'Rel_OC',   'simulink/Logic and Bit Operations/Relational Operator', [270 130 310 160], 'Operator', '>');
    blk(sys, 'Rel_OT',   'simulink/Logic and Bit Operations/Relational Operator', [270 230 310 260], 'Operator', '>');
    blk(sys, 'OR_Any',   'simulink/Logic and Bit Operations/Logical Operator', [400 120 430 200], 'Operator', 'OR', 'Inputs', '4');
    blk(sys, 'OV',       'simulink/Sinks/Out1', [500 28 530 42],   'Port', '1');
    blk(sys, 'UV',       'simulink/Sinks/Out1', [500 88 530 102],  'Port', '2');
    blk(sys, 'OC',       'simulink/Sinks/Out1', [500 138 530 152], 'Port', '3');
    blk(sys, 'OT',       'simulink/Sinks/Out1', [500 238 530 252], 'Port', '4');
    blk(sys, 'ANY',      'simulink/Sinks/Out1', [500 153 530 167], 'Port', '5');
    ln(sys, 'V/1',         'Rel_OV/1');
    ln(sys, 'Vmax_const/1','Rel_OV/2');
    ln(sys, 'V/1',         'Rel_UV/1');
    ln(sys, 'Vmin_const/1','Rel_UV/2');
    ln(sys, 'I/1',         'Abs_I/1');
    ln(sys, 'Abs_I/1',     'Rel_OC/1');
    ln(sys, 'Imax_const/1','Rel_OC/2');
    ln(sys, 'T/1',         'Rel_OT/1');
    ln(sys, 'Tmax_const/1','Rel_OT/2');
    ln(sys, 'Rel_OV/1',    'OR_Any/1');
    ln(sys, 'Rel_UV/1',    'OR_Any/2');
    ln(sys, 'Rel_OC/1',    'OR_Any/3');
    ln(sys, 'Rel_OT/1',    'OR_Any/4');
    ln(sys, 'Rel_OV/1',    'OV/1');
    ln(sys, 'Rel_UV/1',    'UV/1');
    ln(sys, 'Rel_OC/1',    'OC/1');
    ln(sys, 'Rel_OT/1',    'OT/1');
    ln(sys, 'OR_Any/1',    'ANY/1');

    %% =================== Protection_Control (optional) =======================
    % SIMULATED contactor-open command. A fault sets a latch (one-sample delay
    % breaks the algebraic loop); the latch opens the "contactor", i.e. forces
    % the load current to zero. protection_enable = 0 disables it.
    % This is only a signal in a simulation, NOT a hardware disconnection system.
    sys = mdl;
    blk(sys, 'Protection_Control', 'built-in/Subsystem', [240 60 380 160]);
    sys = [mdl '/Protection_Control'];
    blk(sys, 'I_cmd',          'simulink/Sources/In1', [20 40 50 54],    'Port', '1');
    blk(sys, 'Fault_any',      'simulink/Sources/In1', [20 140 50 154],  'Port', '2');
    blk(sys, 'OR_Latch',       'simulink/Logic and Bit Operations/Logical Operator', [100 130 130 170], 'Operator', 'OR', 'Inputs', '2');
    blk(sys, 'Latch_Memory',   'simulink/Discrete/Unit Delay', [170 130 210 170], 'InitialCondition', '0', 'SampleTime', 'Ts');
    blk(sys, 'Prot_enable',    'simulink/Sources/Constant', [170 200 230 230], 'Value', 'logical(protection_enable)', 'OutDataTypeStr', 'boolean');
    blk(sys, 'AND_Enable',     'simulink/Logic and Bit Operations/Logical Operator', [280 140 310 180], 'Operator', 'AND', 'Inputs', '2');
    blk(sys, 'NOT_Trip',       'simulink/Logic and Bit Operations/Logical Operator', [350 145 380 175], 'Operator', 'NOT');
    blk(sys, 'To_Double',      'simulink/Signal Attributes/Data Type Conversion', [420 145 470 175], 'OutDataTypeStr', 'double');
    blk(sys, 'Gate_I',         'simulink/Math Operations/Product', [530 40 560 80], 'Inputs', '2');
    blk(sys, 'I_actual',       'simulink/Sinks/Out1', [620 53 650 67],   'Port', '1');
    blk(sys, 'Contactor_closed', 'simulink/Sinks/Out1', [620 153 650 167], 'Port', '2');
    blk(sys, 'Trip_latched',   'simulink/Sinks/Out1', [620 213 650 227], 'Port', '3');
    ln(sys, 'Fault_any/1',     'OR_Latch/1');
    ln(sys, 'OR_Latch/1',      'Latch_Memory/1');
    ln(sys, 'Latch_Memory/1',  'OR_Latch/2');
    ln(sys, 'Latch_Memory/1',  'AND_Enable/1');
    ln(sys, 'Prot_enable/1',   'AND_Enable/2');
    ln(sys, 'AND_Enable/1',    'NOT_Trip/1');
    ln(sys, 'NOT_Trip/1',      'To_Double/1');
    ln(sys, 'I_cmd/1',         'Gate_I/1');
    ln(sys, 'To_Double/1',     'Gate_I/2');
    ln(sys, 'Gate_I/1',        'I_actual/1');
    ln(sys, 'To_Double/1',     'Contactor_closed/1');
    ln(sys, 'AND_Enable/1',    'Trip_latched/1');

    %% =================== SUBSYSTEM 5: Visualization ==========================
    sys = mdl;
    blk(sys, 'Visualization', 'built-in/Subsystem', [1240 120 1380 420]);
    sys = [mdl '/Visualization'];
    blk(sys, 'SOC_true_in',  'simulink/Sources/In1', [20 20 50 34],    'Port', '1');
    blk(sys, 'SOC_est_in',   'simulink/Sources/In1', [20 70 50 84],    'Port', '2');
    blk(sys, 'V_in',         'simulink/Sources/In1', [20 120 50 134],  'Port', '3');
    blk(sys, 'I_in',         'simulink/Sources/In1', [20 170 50 184],  'Port', '4');
    blk(sys, 'T_in',         'simulink/Sources/In1', [20 220 50 234],  'Port', '5');
    blk(sys, 'OV_in',        'simulink/Sources/In1', [20 270 50 284],  'Port', '6');
    blk(sys, 'UV_in',        'simulink/Sources/In1', [20 300 50 314],  'Port', '7');
    blk(sys, 'OC_in',        'simulink/Sources/In1', [20 330 50 344],  'Port', '8');
    blk(sys, 'OT_in',        'simulink/Sources/In1', [20 360 50 374],  'Port', '9');
    blk(sys, 'ANY_in',       'simulink/Sources/In1', [20 390 50 404],  'Port', '10');
    blk(sys, 'Contactor_in', 'simulink/Sources/In1', [20 440 50 454],  'Port', '11');
    blk(sys, 'Scope_SOC',         'simulink/Sinks/Scope', [200 20 240 90],   'NumInputPorts', '2');
    blk(sys, 'Scope_Voltage',     'simulink/Sinks/Scope', [200 110 240 140]);
    blk(sys, 'Scope_Current',     'simulink/Sinks/Scope', [200 160 240 190]);
    blk(sys, 'Scope_Temperature', 'simulink/Sinks/Scope', [200 210 240 240]);
    blk(sys, 'Scope_Faults',      'simulink/Sinks/Scope', [200 260 240 410], 'NumInputPorts', '5');
    blk(sys, 'Display_SOC',       'simulink/Sinks/Display', [320 60 400 90]);
    blk(sys, 'Display_Fault_Any', 'simulink/Sinks/Display', [320 380 400 410]);
    blk(sys, 'Display_Contactor', 'simulink/Sinks/Display', [320 430 400 460]);
    ln(sys, 'SOC_true_in/1',  'Scope_SOC/1');
    ln(sys, 'SOC_est_in/1',   'Scope_SOC/2');
    ln(sys, 'SOC_est_in/1',   'Display_SOC/1');
    ln(sys, 'V_in/1',         'Scope_Voltage/1');
    ln(sys, 'I_in/1',         'Scope_Current/1');
    ln(sys, 'T_in/1',         'Scope_Temperature/1');
    ln(sys, 'OV_in/1',        'Scope_Faults/1');
    ln(sys, 'UV_in/1',        'Scope_Faults/2');
    ln(sys, 'OC_in/1',        'Scope_Faults/3');
    ln(sys, 'OT_in/1',        'Scope_Faults/4');
    ln(sys, 'ANY_in/1',       'Scope_Faults/5');
    ln(sys, 'ANY_in/1',       'Display_Fault_Any/1');
    ln(sys, 'Contactor_in/1', 'Display_Contactor/1');

    %% =================== TOP LEVEL: sources, stage switch, wiring ============
    sys = mdl;
    blk(sys, 'ILoad_Profile', 'simulink/Sources/From Workspace', [40 60 130 90],   'VariableName', 'I_load_data', 'SampleTime', '0', 'Interpolate', 'on', 'OutputAfterFinalValue', 'Holding final value');
    blk(sys, 'Tamb_Profile',  'simulink/Sources/From Workspace', [40 180 130 210], 'VariableName', 'Tamb_data',   'SampleTime', '0', 'Interpolate', 'on', 'OutputAfterFinalValue', 'Holding final value');
    blk(sys, 'V_Synthetic',   'simulink/Sources/From Workspace', [420 260 510 290], 'VariableName', 'V_sig_data', 'SampleTime', '0', 'Interpolate', 'on', 'OutputAfterFinalValue', 'Holding final value');
    blk(sys, 'I_Synthetic',   'simulink/Sources/From Workspace', [420 320 510 350], 'VariableName', 'I_sig_data', 'SampleTime', '0', 'Interpolate', 'on', 'OutputAfterFinalValue', 'Holding final value');
    blk(sys, 'T_Synthetic',   'simulink/Sources/From Workspace', [420 380 510 410], 'VariableName', 'T_sig_data', 'SampleTime', '0', 'Interpolate', 'on', 'OutputAfterFinalValue', 'Holding final value');
    blk(sys, 'Stage_Select',  'simulink/Sources/Constant',       [420 450 510 480], 'Value', 'stage_select');
    blk(sys, 'Switch_V', 'simulink/Signal Routing/Switch', [620 90 660 130],  'Criteria', 'u2 >= Threshold', 'Threshold', '0.5');
    blk(sys, 'Switch_I', 'simulink/Signal Routing/Switch', [620 190 660 230], 'Criteria', 'u2 >= Threshold', 'Threshold', '0.5');
    blk(sys, 'Switch_T', 'simulink/Signal Routing/Switch', [620 290 660 330], 'Criteria', 'u2 >= Threshold', 'Threshold', '0.5');
    ln(sys, 'ILoad_Profile/1',     'Protection_Control/1');
    ln(sys, 'Protection_Control/1','Battery_Model/1');
    ln(sys, 'Tamb_Profile/1',      'Battery_Model/2');
    ln(sys, 'Battery_Model/1',     'Switch_V/1');
    ln(sys, 'V_Synthetic/1',       'Switch_V/3');
    ln(sys, 'Stage_Select/1',      'Switch_V/2');
    ln(sys, 'Battery_Model/2',     'Switch_I/1');
    ln(sys, 'I_Synthetic/1',       'Switch_I/3');
    ln(sys, 'Stage_Select/1',      'Switch_I/2');
    ln(sys, 'Battery_Model/3',     'Switch_T/1');
    ln(sys, 'T_Synthetic/1',       'Switch_T/3');
    ln(sys, 'Stage_Select/1',      'Switch_T/2');
    ln(sys, 'Switch_V/1',          'Battery_Monitoring/1');
    ln(sys, 'Switch_I/1',          'Battery_Monitoring/2');
    ln(sys, 'Switch_T/1',          'Battery_Monitoring/3');
    ln(sys, 'SOC_Estimator/1',     'Battery_Monitoring/4');
    ln(sys, 'Battery_Monitoring/2','SOC_Estimator/1');
    ln(sys, 'Battery_Monitoring/1','Fault_Detection/1');
    ln(sys, 'Battery_Monitoring/2','Fault_Detection/2');
    ln(sys, 'Battery_Monitoring/3','Fault_Detection/3');
    ln(sys, 'Fault_Detection/5',   'Protection_Control/2');
    ln(sys, 'Battery_Model/4',     'Visualization/1');
    ln(sys, 'Battery_Monitoring/4','Visualization/2');
    ln(sys, 'Battery_Monitoring/1','Visualization/3');
    ln(sys, 'Battery_Monitoring/2','Visualization/4');
    ln(sys, 'Battery_Monitoring/3','Visualization/5');
    ln(sys, 'Fault_Detection/1',   'Visualization/6');
    ln(sys, 'Fault_Detection/2',   'Visualization/7');
    ln(sys, 'Fault_Detection/3',   'Visualization/8');
    ln(sys, 'Fault_Detection/4',   'Visualization/9');
    ln(sys, 'Fault_Detection/5',   'Visualization/10');
    ln(sys, 'Protection_Control/2','Visualization/11');

    %% =================== Logging to workspace (for post-processing) ==========
    blk(sys, 'log_SOC_true', 'simulink/Sinks/To Workspace', [1440 20 1520 40],  'VariableName', 'log_SOC_true', 'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    blk(sys, 'log_SOC_est',  'simulink/Sinks/To Workspace', [1440 60 1520 80],  'VariableName', 'log_SOC_est',  'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    blk(sys, 'log_V',        'simulink/Sinks/To Workspace', [1440 100 1520 120], 'VariableName', 'log_V',       'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    blk(sys, 'log_I',        'simulink/Sinks/To Workspace', [1440 140 1520 160], 'VariableName', 'log_I',       'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    blk(sys, 'log_T',        'simulink/Sinks/To Workspace', [1440 180 1520 200], 'VariableName', 'log_T',       'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    blk(sys, 'log_OV',       'simulink/Sinks/To Workspace', [1440 220 1520 240], 'VariableName', 'log_OV',      'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    blk(sys, 'log_UV',       'simulink/Sinks/To Workspace', [1440 260 1520 280], 'VariableName', 'log_UV',      'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    blk(sys, 'log_OC',       'simulink/Sinks/To Workspace', [1440 300 1520 320], 'VariableName', 'log_OC',      'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    blk(sys, 'log_OT',       'simulink/Sinks/To Workspace', [1440 340 1520 360], 'VariableName', 'log_OT',      'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    blk(sys, 'log_ANY',      'simulink/Sinks/To Workspace', [1440 380 1520 400], 'VariableName', 'log_ANY',     'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    blk(sys, 'log_Contactor','simulink/Sinks/To Workspace', [1440 420 1520 440], 'VariableName', 'log_Contactor', 'SaveFormat', 'Array', 'MaxDataPoints', 'inf');
    ln(sys, 'Battery_Model/4',     'log_SOC_true/1');
    ln(sys, 'Battery_Monitoring/4','log_SOC_est/1');
    ln(sys, 'Battery_Monitoring/1','log_V/1');
    ln(sys, 'Battery_Monitoring/2','log_I/1');
    ln(sys, 'Battery_Monitoring/3','log_T/1');
    ln(sys, 'Fault_Detection/1',   'log_OV/1');
    ln(sys, 'Fault_Detection/2',   'log_UV/1');
    ln(sys, 'Fault_Detection/3',   'log_OC/1');
    ln(sys, 'Fault_Detection/4',   'log_OT/1');
    ln(sys, 'Fault_Detection/5',   'log_ANY/1');
    ln(sys, 'Protection_Control/2','log_Contactor/1');

    %% =================== Save, compile-check, optional test run ==============
    save_system(mdl, [mdl '.slx']);
    fprintf('Saved %s.slx\n', mdl);

    try
        set_param(mdl, 'SimulationCommand', 'update');      % compile check
        fprintf('Model diagram update (compile check): OK\n');
    catch ME
        fprintf(2, 'Compile check FAILED:\n%s\n', ME.message);
        fprintf(2, 'See the troubleshooting section of the guide.\n');
        return;
    end

    if run_test
        try
            out = sim(mdl);
            soc = out.get('log_SOC_est');
            fprintf('Test run OK. Final estimated SOC = %.2f %% (scenario %d).\n', ...
                    soc(end), evalin('base', 'scenario_id'));
        catch ME
            fprintf(2, 'Test run FAILED:\n%s\n', ME.message);
        end
    end
    save_system(mdl);
end

% ---------------------------------------------------------------------------
function blk(sys, name, lib, pos, varargin)
%BLK  add_block wrapper: library path, name, [x1 y1 x2 y2] position, params.
    add_block(lib, [sys '/' name], 'Position', pos, varargin{:});
end

function ln(sys, src, dst)
%LN  add_line wrapper: 'Block/port' -> 'Block/port'.
    add_line(sys, src, dst, 'autorouting', 'on');
end

### 6.3 verify_simulink_vs_matlab.m

%% verify_simulink_vs_matlab.m
%  Runs EV_BMS_SOC.slx for scenarios 1..11 and compares it with the MATLAB
%  simulation (simulate_bms). Run create_EV_BMS_model first.
%
%  Expected differences (not bugs):
%   * Simulink uses forward-Euler integration, so its SOC at step k uses the
%     current up to step k-1, while coulomb_counting() uses I(k). The final SOC
%     differs by at most one sample of charge: I*dt/(3600*Q)*100 (about 0.04 %
%     for 100 A, 1 s, 50 Ah).
%   * Fault flags may start/stop one sample later in Simulink for the same reason.

clear; clc;
mdl = 'EV_BMS_SOC';
if ~bdIsLoaded(mdl), load_system(mdl); end
P = bms_params();

fprintf('%-3s %-32s %10s %10s %10s  %s\n', '#', 'Scenario', 'SOC_matlab', 'SOC_simulink', 'diff(%)', 'faults match');
for id = 1:11
    assignin('base', 'scenario_id', id);
    assignin('base', 'stage_select', 1);
    evalin('base', 'bms_simulink_init;');
    sc = bms_scenarios(id, P);
    R  = simulate_bms(P, sc);

    out = sim(mdl);
    soc_sl = out.get('log_SOC_est');
    flags_sl = [any(out.get('log_OV')) any(out.get('log_UV')) ...
                any(out.get('log_OC')) any(out.get('log_OT'))];
    flags_ml = [any(R.OV) any(R.UV) any(R.OC) any(R.OT)];
    fprintf('%-3d %-32s %10.2f %10.2f %10.3f  %s\n', id, sc.name, ...
        R.SOC_est(end), soc_sl(end), soc_sl(end) - R.SOC_est(end), ...
        mat2str(isequal(flags_sl, flags_ml)));
end
fprintf('\nIf SOC differs by more than about 0.1 %% or the fault flags do not match, see Troubleshooting.\n');

## 7. Instructions to run the project

- Copy all .m files into one folder and make it the current folder in MATLAB.

- MATLAB only: >> EV_BMS_Main. The console shows the Stage A check, a table for scenarios 1–11 with PASS/FAIL, first-fault times, the manual SOC check and the sensor-offset demo. Figures open for scenarios 3 and 9. Change PLOT_IDS at the top to see others.

- Simulink: >> create_EV_BMS_model. It saves EV_BMS_SOC.slx and runs a test. To pick a scenario:

- scenario_id = 6;  stage_select = 1;   % Stage B, overcurrent
sim('EV_BMS_SOC');                    % then open the scopes in Visualization

- For Stage A: stage_select = 0. Double-click Visualization and open each Scope.

- >> verify_simulink_vs_matlab compares both implementations for all scenarios.

- Optional: >> EV_BMS_EKF_demo.

- To explore: edit values in bms_params.m (capacity, thresholds, I_offset, SOC_est_init) and rerun. Rerun create_EV_BMS_model only if you change the structure of the model.

Replacing the illustrative OCV curve: rest a cell for 1–2 h at each SOC step (for example every 10 %), record the settled voltage, and paste the numbers into P.SOC_bp and P.OCV_cell. Estimate R0 from the instantaneous voltage step of a current pulse, R0 = ΔV/ΔI. Do this per temperature for a real design.

## 8. Test cases and expected behaviour

Scenarios 1–10 run with the simulated contactor off, so every flag stays visible for the whole run. Scenario 11 runs it on. Expected values below come from the Python cross-check of the same equations.

| # | Scenario | Inputs | Expected behaviour | Flags expected | Pass criterion |
| --- | --- | --- | --- | --- | --- |
| 1 | Normal discharge | SOC₀ 80 %, 20 A, 1800 s | SOC falls linearly to 60 %. V from about 52.0 V down to 49.6 V. T about 26 °C. | none | final SOC = 60.00 ± 0.05; no flag |
| 2 | Charging | SOC₀ 30 %, −25 A, 3600 s | SOC rises linearly to 80 %. V rises to about 53.1 V. | none | final SOC = 80.00 ± 0.05; V stays below 54.6 V |
| 3 | Variable load | SOC₀ 90 %, piecewise profile peak 70 A with regen | SOC falls with dips back up during regen. Final about 46.7 %. V 48.4–53.7 V. | none | no flag; SOC moves with the sign of I |
| 4 | Overvoltage | SOC₀ 85 %, −40 A, 600 s | V climbs above 54.6 V. | OV from about 191 s | OV asserted, others not |
| 5 | Undervoltage | SOC₀ 12 %, +60 A, 600 s | SOC reaches 0 % at about 360 s. V falls below 39 V from about 303 s. | UV | UV asserted, others not; SOC stays at 0 |
| 6 | Overcurrent (discharge) | SOC₀ 70 %, 20 A with 120 A pulse 300–400 s | Current pulse crosses 100 A. | OC for 300–399 s | OC asserted only in the pulse |
| 7 | Excessive charge current | SOC₀ 40 %, −20 A with −120 A pulse 300–400 s | abs(I) > I_max also catches charging. | OC for 300–399 s | OC asserted only in the pulse |
| 8 | Overtemperature | SOC₀ 95 %, T₀ 50 °C, T_amb 40 °C, 80 A, 1800 s | I²R heating pushes T through 60 °C and up to about 61 °C. | OT from about 1503 s | OT asserted, others not |
| 9 | Multiple faults | SOC₀ 95 %, T₀ 58 °C, T_amb 45 °C, −130 A, 300 s | V about 57.4 V and ¦I¦ = 130 A from the first sample. T crosses 60 °C at about 49 s. | OV + OC from 0 s, OT from about 49 s | three flags asserted, UV not |
| 10 | SOC near lower limit | SOC₀ 15 %, +30 A, 1200 s | SOC reaches 0 % at 900 s and saturates. V drops below 39 V from about 843 s. | UV | final SOC = 0.00; UV asserted |
| 11 | Protection demo | As scenario 6, contactor enabled | OC at 300 s; contactor opens at the next sample and stays open (latched). Current becomes 0. | OC (once) | contactor open at end; I(end) = 0; SOC stops falling |

EV_BMS_Main performs these checks automatically and prints PASS or FAIL per scenario.

Stage A windows (seconds): OV 100–150 and 560–600; UV 200–250; OC 300–350, 400–450 and 560–600; OT 500–550 and 560–600. Everything else is normal.

## 9. Results analysis

How to read the plots

- SOC vs time: the true and estimated curves overlap in the base case. Slope is proportional to current: discharge falls, charge rises, zero current is flat. After saturation at 0 or 100 % the curve is flat.

- Voltage: steps with each current step, because of I·R0: a 100 A discharge pulse drops about 2.5 V instantly. The slow trend follows SOC through the OCV curve.

- Current: positive = discharge, negative = charge. The dashed lines are ±I_max.

- Temperature: rises as I² (heating grows with the square of current) and relaxes toward ambient with time constant Rth·Cth = 1200 s.

- Fault flags: each flag is a 0/1 stair trace. Check the time it rises against the threshold crossing in the corresponding signal.

- Combined fault: high whenever any flag is high. With the contactor enabled, the green trace shows it opening.

Manual SOC verification

- Scenario 1: ΔSOC = 20 A × 1800 s / (3600 × 50 Ah) = 0.20 → 20 %, so 80 − 20 = 60 %.

- Scenario 2: ΔSOC = 25 × 3600 / (3600 × 50) = 0.50 → +50 %, so 30 + 50 = 80 %.

- Simulink gain: −100/(3600 × 50) = −5.5556×10⁻⁴. For 20 A over 1800 s: −5.5556×10⁻⁴ × 20 × 1800 = −20 %. ✔

Sources of discrepancy

| Cause | Effect | Example |
| --- | --- | --- |
| Wrong initial SOC | Constant offset that never decays | Start at 60 % instead of 80 %: error stays 20 % |
| Current-sensor offset | Error grows with time (drift) | +2 A offset for 1800 s: 2×1800/(3600×50) = 2 % error (the main script prints this) |
| Wrong capacity | Error proportional to the SOC change | Real 45 Ah vs 50 Ah assumed: a true 22.2 % drop is estimated as 20 % |
| Sampling time | Rectangle-rule error; matters only if current changes within a step | Fine at 1 s for this profile; large dt with fast current changes loses accuracy |
| Euler / indexing | One-sample shift between Simulink and MATLAB | At most I·dt/(3600 Q)·100 ≈ 0.04 % at 100 A |
| No aging, temperature or hysteresis | Model error vs real cell | Not visible in simulation; dominant in real hardware |

Expected numerical results (from the Python cross-check)

| # | SOC_end (%) | V_min (V) | V_max (V) | T_max (°C) | First fault times (s) |
| --- | --- | --- | --- | --- | --- |
| 1 | 60.00 | 49.55 | 52.02 | 26.2 | none |
| 2 | 80.00 | 47.95 | 53.14 | 27.2 | none |
| 3 | 46.67 | 48.40 | 53.69 | 29.0 | none |
| 4 | 98.33 | 54.11 | 55.45 | 27.4 | OV 191 |
| 5 | 0.00 | 37.50 | 43.66 | 30.3 | UV 303 |
| 6 | 57.78 | 47.05 | 50.72 | 29.6 | OC 300 |
| 7 | 52.22 | 48.60 | 52.01 | 29.6 | OC 300 |
| 8 | 15.00 | 43.63 | 52.14 | 60.9 | OT 1503 |
| 9 | 100.00 | 57.39 | 57.85 | 69.1 | OV 0, OC 0, OT 49 |
| 10 | 0.00 | 38.25 | 44.88 | 27.1 | UV 843 |
| 11 | 66.61 | 47.82 | 50.82 | 25.4 | OC 300 (contactor opens at 301) |

These are not MATLAB outputs. Run EV_BMS_Main and compare. A difference beyond rounding means a typo or a changed parameter.

Limits of the simulation. The model reproduces how the algorithms behave, not how a real pack behaves. The OCV curve, R0, Rth and Cth are illustrative, so absolute voltages and temperatures are not predictions for any real battery.

## 12. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Undefined function 'bms_params' | Files not in the current folder or path | cd to the project folder, or addpath(pwd) |
| Undefined function or variable 'k_soc' (or other name) during sim | Init script did not run | evalin('base','bms_simulink_init;') or run bms_simulink_init at the command line |
| new_system / add_block error about an invalid library path | Block path differs in your release | In Simulink Library Browser, right-click the block → Copy path, replace the string in the failing blk(...) line |
| Invalid setting ... parameter 'ExtrapMethod' or 'BreakpointsForDimension1' | Release uses different parameter names for the lookup block | Run get_param('EV_BMS_SOC/Battery_Model/OCV_Lookup','DialogParameters') after adding the block by hand, use the names it lists for the breakpoints and table, and delete the ExtrapMethod line. Or open the block dialog and type SOC_bp and OCV_pack into it manually |
| Invalid setting 'NumInputPorts' on a Scope | Very old Scope block | Replace the multi-port scope with a Mux and a one-input scope, or one scope per signal |
| Block diagram contains an algebraic loop | Latch delay missing or its sample time wrong | Check Latch_Memory exists with sample time Ts; set Ts in base workspace |
| Boolean/double data-type error at Logical Operator, Product or Switch | Data-type strictness in your release | Check that Prot_enable has Output data type boolean, and that To_Double outputs double |
| “Fixed step size is too large” or integrator warnings | Step size different from Ts | Set Solver to ode1 fixed-step with Fixed step size Ts |
| From Workspace error “time must be strictly monotonically increasing” | Edited P.dt to 0 or duplicated times | Keep P.dt > 0; rerun bms_simulink_init |
| sim returns no log_* variables | ReturnWorkspaceOutputs off | Use out = sim('EV_BMS_SOC'); out.get('log_SOC_est'), or the variable appears in the base workspace in older releases |
| SOC in Simulink differs by more than about 0.1 % from MATLAB | Gain wrong, or Q_Ah changed after init | Print k_soc; it must equal -100/(3600*Q_Ah) |
| Scopes empty | Visualization subsystem not opened, or simulation not run | Run the model, then double-click each scope |
| EV_BMS_Main prints FAIL for a scenario | A parameter in bms_params.m changed | Compare with the default thresholds in section 2; scenarios are tuned to the defaults |
| Function-at-end-of-script error in EV_BMS_Main | MATLAB older than R2016b | Move passfail into its own file passfail.m |
| ode1 unavailable | Not a standard release | Any fixed-step solver works; set fixed-step size Ts |

If something still fails, send the exact error text and your MATLAB version (ver). That is enough to locate the line.

## 13. Future improvements

- EKF SOC estimation validated against reference data (section 3.10).

- Cell balancing (passive bleed resistors first, then active).

- State of Health estimation (capacity fade, resistance growth).

- Fault debounce timers and hysteresis; staged derating before a hard trip.

- Temperature- and SOC-dependent R0; 2-RC model; thermal model with cooling.

- Pre-charge logic, contactor weld detection, insulation monitoring.